Material Receiving

Specifications

Trucks

All materials must be on pallets. All deliveries must be made with a dock high truck. All material must be able to be unloaded with a pallet jack.  The warehouses will only receive a maximum of 10 cartons that are not on skids.

Cartons

All materials must be packed in cartons. Size should conform as closely as possible to the contents.

Test Strength:   	200 pounds (minimum)

Gross Weight:   	Not to exceed 40 pounds

Identifications:  	Each carton must be identified on the outside with mailer name, program name, keycode, month of collation and quantity in carton. Please attach a sample of the component to the outside of the box.

##### BULK PACKED OR TELESCOPED CARTONS WILL ONLY BE ACCEPTED IF THE COMPONENT WILL BE USED WITHIN 30 DAYS.

##### DO NOT MIX CODES/ VERSIONS WITHIN CARTONS

##### ALL MATERIAL MUST FACE THE SAME DIRECTION IN THE CARTON OR BE CLEARLY SEPARATED BY CHIPBOARD

Pallets

Slip sheeted or bed loaded trailers will not be accepted. 40” x 48” four way wooden pallets in good condition should be used. We prefer to receive rolls “pancake” style and not “roll off” style.

Dimensions:       	Not to exceed 48” in length. Open side (fork side) must have a dimension of 40”. Height must not exceed 58” including a pallet.

Weight:	Not to exceed 2,500 pounds.

Securing Load:	Material must be secured to pallet in a manner such as to inhibit shifting and crushing of contents.  Banding with plastic, stretch wrapping and shrink-wrapping are acceptable methods.  Plastic bands must be a minimum ½” wide, 600# test embossed strapping.  Metal strapping is prohibited.

###### DOUBLE STACKED PALLETS WILL NOT BE ACCEPTED

Identification

Each pallet must contain ONLY one code/version unless arranged by Planner and Vendor.

Packing List

A packing list must accompany each shipment outlining the following minimal information.

- Mailer Name

- Monoprice PIP

- Number of cartons by mailer name and keycode

- Total number of skids

- Month of collation

Address & Receiving Hours

Mark Kibler – 80% of the order quantity

11701 6th Street

Rancho Cucamonga, CA 91730

909-912-1371

Mark Kibler – 20% of the order quantity

2055 Global Way

Hebron, KY 41048

909-912-1371

#### Air Freight

Cartons should be reinforced when shipping by air by carriers who will deliver directly to the warehouses.

Rejection of a Delivery

Material Control can reject a delivery of components or charge back** the vendor (or customer) if the following occur:

- Skids are over 58” high.

- Damage that affects or could affect the usability of components or forms.

- Damage so that the skids cannot be unloaded by fork truck or pallet jack.

- Packing List that does not match what is actually received by J&Q.

- Box labels that do not match carton count or component code.

- Water damage of any cartons or rolls.

- No more than 10 cartons not on skids.  (Maximum 40 pounds each)

- Any truck that is not at dock level delivering skids.

- Any components shipped overnight/messenger that do not have a packing list.  Each carton must be clearly labeled with:

- Mailer Name

- Monoprice PIP

- Number of cartons by mailer name and keycode

- Total number of skids

- Month of collation

t

**Material Control will charge the vendor (or customer) a one-time administration fee of

$100 per delivery, plus $50/hour to conform a delivery to specifications.